Conexant Systems, Inc.

RipTide-PCI driver release notes for Windows NT 4.0

1) Introduction

This is Beta release, version 4.03.2222 (internal version is 2.22), of Windows NT 4.0 Audio Driver for RipTide Audio PCI device.

2) Hardware and Software Requirement

Pentium PC 200Mhz with at least 16M RAM (more than 32M is recommended)

Windows NT 4.0 Service Pack 5

RipTide Hardware D7400 or D7300

3) Installation Procedure

If RipTide-PCI board is not installed yet, you need install it:

- Exit your Windows NT

- Power Off your PC

- Open your PC, and insert the RipTide-PCI board in one of available PCI slots

- Power your PC up

Now RipTide-PCI board is installed.

Verify BIOS configuration:

If BIOS has “Plug & Play O/S” or similar setting, it must be set to “No”, because Windows NT 4.0 is not Plug & Play Operating system.

If the setting is wrong, Windows NT 4.0 may sometimes have problems with PCI devices.

If old version is installed, you need remove it:

- Click Start icon on your Windows

- Select Control Panels from Settings

- Double click on “Add/Remove Programs” icon on Control Panel window

- Select Riptide Line in the Install Software Package List

- Click Remove

- If the machine does not reboot automatically, restart the machine

Now old RipTide-PCI drivers are removed from your machine.

Install driver:

- Open the CDROM Icon in explorer or from My Computer

- Run SETUP.EXE distributive

- Click “Next” as required until the setup is complete

- If the machine does not reboot on its own, restart the machine

Riptide is now installed.

Note: if it is combo release (audio and modem), this procedure will install drivers for both devices.

Also, audio uninstall always removes modem driver.

4) Release Content

_INST32I.EX_			Setup file

_ISDEL.EXE			Setup file

_SETUP.DLL			Setup file

_SYS1.CAB			Setup file

_USER1.CAB			Setup file

DATA.TAG			Setup file

DATA1.CAB			Setup file

DELTA.TMP			Setup file

LANG.DAT			Setup file

LAYOUT.BIN			Setup file

OS.DAT			Setup file

SETUP.BMP			Setup file

SETUP.EXE			Setup file

SETUP.INI			Setup file

SETUP.INS			Setup file

SETUP.ISS			Setup file

SETUP.LID			Setup file

AUDIO.NT4\OEMSETUP.INF	Main installation script file

AUDIO.NT4\RIPTIDE.INF	Rockwell script file

AUDIO.NT4\RPDLL32.DLL	RipTide Dynamic Link Library

AUDIO.NT4\WSDLL32.DLL	WaveStream Dynamic Link Library

AUDIO.NT4\MSG32NT.EXE	WaveStream message loop

AUDIO.NT4\RIPTIDE.SYS	RipTide Kernel Driver

AUDIO.NT4\WSDRIVER.SYS	WaveStream Kernel Driver

AUDIO.NT4\D7400.HEX	Firmware downloadable binary for D7400 board

AUDIO.NT4\D7300.HEX	Firmware downloadable binary for D7300 board

AUDIO.NT4\WSTREAM.BIN	WaveStream Sound library

AUDIO.NT4\GENMIDI.WSL	WaveStream Sound library

AUDIO.NT4\WSTREAMS.BIN	WaveStream Sound library (small)

AUDIO.NT4\GENMIDIS.WSL	WaveStream Sound library (small)

AUDIO.NT4\WSMONITR.EXE	WaveStream monitor

HCF.NT4\*.*			HCF driver files (Combo distribution only)

HSF.NT4\*.*			HSF driver files (Combo distribution only)

5) Release History

Release 2.22

Features/Bugs Fixed:

- Fixed resources allocation. Error message ("The description for Event ID ( 22 ) in Source ( riptide ) could not be found. It contains the following insertion string(s): , riptide, \Device\RipTideMidiOut0.Translated.") will not appear in Windows NT event log.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

Changes:

- RIPTIDE.SYS

- Other *.EXE, *.DLL, *.SYS – Version is changed.

Release 2.21

Features/Bugs Fixed:

- Now driver can work properly on computers with 2 or more PCI buses. Fixed crash when PhoenixBios 6.0 is configured for “Windows 95” or “Windows 98/Windows 2000”. However driver can not work with these settings, because Windows NT does not work properly itself.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

Changes:

- RIPTIDE.SYS

- Other *.EXE, *.DLL, *.SYS – Version is changed.

Release 2.20

Features/Bugs Fixed:

- Fixed cases 823 and 833 for new ACT 4.3.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

Release 2.19

Features/Bugs Fixed:

- Fixed timing problem, which arises on some HP Pavilion computers.

- New firmware is included: 3.8P3.8 (D7400) and 3.9P1.4 (D7300).

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

Release 2.18

Features/Bugs Fixed:

- Now Windows NT 5.0 beta 3 does not lock up during upgrade of Windows NT 4.0 if Riptide drivers installed. (Of course, this is bug of Windows NT 5.0 beta 3, but we can not wait Microsoft to fix it.)

- Now WaveStream is working fine after upgrade of Windows NT 4.0 with another version of Windows NT 4.0.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

- Release 2.17

- Features/Bugs Fixed:

- Fixed WaveStream. It did not work on Korean Windows NT 4.0. (Probably some other national versions were affected too.)

- Updated D7400 firmware to 3.8P3.4.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 3.

- Release 2.16C

- Features/Bugs Fixed:

- WaveStream does not lose sound, when user move the progress slider on mplayer2.exe.

- Fixed setup crash on computers with fresh Winodws NT 4.0 + Service Pack 4

- Improved ACT on D7300.

- Fix for microphone not muting after reboot.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 2.

- Release 2.16

- Features/Bugs Fixed:

- Added D7300 support.

- Completely redesigned driver set-up to be more reliable.

- Fixed problems with latest Microsoft ACT 4.1 (work around: firmware reports completion of block before it really completed).

- Fixed WaveStream bug (Sometimes WaveStream is disabled after installation).

- Updated firmware: now driver is using 3.8P3.2 for D7400 and 3.9P1.2 for D7300.

- "Rockwell Semiconductor Systems" is changed to "Conexant Systems, Inc."

- Limitations:

- Driver was not tested with D7300 board. Probably some ACT tests may fail on D7300.

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 2.

# Release 2.15

- Features/Bugs Fixed:

- New firmware.

- Something with master mute. Sorry, I do not know exactly.

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 2.

# Release 2.14

- Features/Bugs Fixed:

- Fixed lockup problem with dual CPU computers.

- Improved mixer (association between application wave channel and wave volume control).

- Limitations:

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- WaveStream does not work on Windows NT 5.0 beta 2.

# Release 2.13

- Features/Bugs Fixed:

- Fixed problem with firmware timing.

- Firmware is updated to V3.8P1.9.

# Release 2.12

- Features/Bugs Fixed:

- Updated to support firmware V3.8P1.8.

- Fixed driver problem with installation over failed or damaged installation.

- Changed version and product name for WaveStream components.

# Release 2.11a

- Features/Bugs Fixed:

- Updated to support firmware V3.8P1.6.

# Release 2.11

- Features/Bugs Fixed:

- Fixed ACT98 test case 37.

# Release 2.10

- Features/Bugs Fixed:

- Firmware is upgraded.

- Implemented work around against FM\OPL firmware problem.

# Release 2.09

Features/Bugs Fixed:

- Since this release driver support D7400 only. It will work with D7501 if you replace FIRMWARE.HEX with firmware for D7501.

- Changes in installation/configuration.

- Changed version to be adequate Windows 95 driver’s version.

- Some minor changes to improve compatibility with Windows NT 5.0 beta 2.

- Version is changed to 4.03.2209 (internal version is 2.09). It is because major version should be non-zero. Version 1.XX is reserved for Windows 95.

- Limitations (the following bugs will be fixed in next release):

- Enable/Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- MIDI Driver Test Case 1 “MIDM_GETDEVCAPS” failed due to non-zero dwSupport. We believe that it is problem of WinNT multimedia system. The problem does not exist on Windows NT 5.0 for the same driver.

# Release 0.08

- Features/Bugs Fixed:

- Change of volume and mute is smooth now.

- Performance of FM/OPL3 was improved.

- Performance of MPU401 OUT PORT was improved.

- Changed default mixer settings.

- Changes in installation/configuration.

- Added new product IDs. Some ACT Tests will report that product ID is not registered by Microsoft. Actually, products IDs were registered some days before. So ACT Tests do not know about these IDs. It is OK. See WHQL PC98 Audio “Appendix C: Common failures” for details.

- Firmware version is 3.8.

- Limitations (the following bugs will be fixed in next release):

- Enable\Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- MIDI Driver Test Case 1 “MIDM_GETDEVCAPS” failed due to non-zero dwSupport. We believe that it is problem of WinNT multimedia system.

# Release 0.07

- Features/Bugs Fixed:

- Many fixes (about 10) in MIDI and WAVE subsystems to be PC98 ACT compatible.

- Added I2S support.

- Redesigned mixer. Now there are 3 panels only.

- Changes in installation/configuration

- Fixed distortions when driver records mono sound with 48 kHz sample rate.

- Fixed distortions if driver record with sample rates below 8 kHz.

- Firmware version is 3.7.

- Limitations (the following bugs will be fixed in next release):

- Enable \ Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- MIDI Driver Test Case 1 “MIDM_GETDEVCAPS” failed. We believe that it is problem of WinNT multimedia system.

# Release 0.06A

- Features/Bugs Fixed:

- Support HSF MMX Modem.

- You may start WaveStream monitor from configuration dialog:

- Go to control panel.

- Start “Multimedia” applet.

- Go to page “Devices”.

- Select one of Riptide devices and push “Properties” button.

- Push setting button. Riptide configuration dialog appears.

- Go to page “WaveStream” and push “WS Monitor” button.

Or start it from command line: WSMONITR.EXE.

- Now it is possible to change WaveStream sampling rate without reboot.

- Some serious fixes in installation process.

- Limitations (the following bugs will be fixed in next release):

- The Uninstall does not remove RPDLL32.DLL.

- Enable\Disable WaveStream requires reboot. Also disabled WaveStream device is detected as used by another application.

- Some ACT tests are failed.

# Release 0.06

- Features/Bugs Fixed:

- Added support for HCF DSP and HSF MMX Modem.

- Included install and uninstall utility.

- Mixer’s Volume controls’ text string can be customized via .INF file.  User should use an UNICODE editor like Microsoft Word, to edit the file.

- Included WaveStream monitor.

- MPU401 performance is greatly improved.

- Bug #290, “ACT Combined Failure Problem” is fixed.

- Bug #298, Disable WaveStream, Play MIDI file, Get blue screen, is fixed.

- Bug #301, WaveStream’s Settings - applet Panel will lock-up when using the [X] button to cancel the applet, is fixed.

- Bug #302, Enable Wave Stream without rebooting, play MIDI file, will get blue screen, is fixed.

- Limitations (the following bugs will be fixed in next release):

- Bug #289, Distorted (Crackling and Popping) audio. Playing multiple (four or more) Wave or AVI Files, is not fixed..

- Bug #293, Audio "Record Mixer Panel" left and right balance controls has very bad stereo separation.

- Bug #299, Change WaveStream sampling rate. Sound distorted.

# Release 0.05

- Features/Bugs Fixed:

- Windows NT WHQL Tests. Fixed Mixer Test cases: 10, 34, 36, 38 and 40.

- Windows NT WHQL Tests. Fixed Wave out Tests case: 5.

- Windows NT WHQL Tests. Fixed Direct Sound Buffer Test issue.

- Windows NT WHQL Tests. Fixed Direct Sound 3D Test issue.

- Windows NT WHQL Tests. Fixed Wave out Tests case: 5.

- Tuned Bass Boost and Stereo enhance.

- Fixed volume controls for CD-input, Line-In and External-Balance.

- Fixed FM/OPL3.

- Redesigned configuration and setup dialog.

- Support PCI device Ids: 0x4300,0x4310,0x4320,0x4330 and 0x4301(optional) .

- Limitations:

- If version 0.01-0.03 is installed, it should be removed before installation. Between removing old driver and installing new driver, use regedt32 to remove the three entries 1 under HKLM\System\CurrentControlSet\Control\MediaResources\MIDI. After this and a fresh installation restart, should come up with WaveStream instead of an extra OPL3.

- Windows NT WHQL Tests. Direct Sound Driver Tests fail cases: 10 and 18. ESS and Yamaha sound cards have the same problem. Probably, it is bug in Windows NT WHQL Tests.

- Windows NT WHQL Tests. WaveOut Tests fail case: 64. ESS and Yamaha sound cards have the same problem. Probably, it is bug in Windows NT WHQL Tests.

- Windows NT WHQL Tests. Mixer Tests fails case: 18. It should be fixed in next release.

- Windows NT WHQL Tests. MIDI Input Driver Tests fails case: 7. It should be fixed in next release.

- Using midi player to open big MIDI file, the latency of getting the file loaded is quite long. It is by design of software synthesizer.

# Release 0.04

- Features/Bugs Fixed:

- Fixed problem with DirectX applications.

- Reduced number of clicks when PCM Wave is started and stopped.

- Disabled configuration dialog.

- Fixed PCI code.

- Firmware upgraded to version 2.9.

- Limitations:

- If previous version is installed, it should be removed before installation. Between removing old driver and installing new driver, use regedt32 to remove the three entries 1 under HKLM\System\CurrentControlSet\Control\MediaResources\MIDI. After this and a fresh installation restart, should come up with WaveStream instead of an extra OPL3.

- Windows NT WHQL Tests. Mixer Test fails cases: 10, 34, 36, 38 and 40. It is happen, because Mixer device returns wrong information about WaveStream volume control. It should be fixed in next release. This problem should not affect functionality.

- Windows NT WHQL Tests. Direct Sound Driver Tests fail cases: 10 and 18. ESS and Yamaha sound cards have the same problem. Probably, it is bug in Windows NT WHQL Tests.

- Windows NT WHQL Tests. Wave Out Tests fail case: 64. ESS and Yamaha sound cards have the same problem. Probably, it is bug in Windows NT WHQL Tests.

- Windows NT WHQL Tests. Wave out Tests fails case: 5. It should be fixed in next release.

- Bass Boost and Stereo enhance have a little effect. It is due to default values of HKLM\System\CurrentControlSet\Services\Riptide\Parameters\Device0\Coeff3D and HKLM\System\CurrentControlSet\Services\Riptide\Parameters\Device0\CoeffBass. Values may be changed manually.

- Balance controls (Right channel) may not work properly on CD-input, Line-In and External-Balance.

- Using midi player to open big MIDI file, the latency of getting the file loaded is quite long. It is by design of software synthesizer.

- Scratchy sound moving master volume slider up and down, left to right, right to left. It may be hardware problem.

- OPL3 does not produce any sound.

# Release 0.03

- Features/Bugs Fixed:

- Added firmware loader.

- Fixed both MPU401 input and output devices.

- Added WaveStream64  support.

- Completely reworked mixer device.

- Reworked setup and installation sequence.

- Fixed dynamic allocation of hardware.

- Fixed problem with mono sounds.

- Fixed PCI code for legacy devices.

- Limitation:

- Problems when DirectX and wave are playing simultaneously.

- Setup does not change WaveStream configuration. It should be fixed in next release.

- If previous version is installed, it should be removed before installation.

# Release 0.02

- Features/Bugs Fixed:

- Added support FM/OPL3.

- Fixed PCM/Wave recording.

- WHQL case #10 (mixerGetLineInfo) is fixed.

- WHQL case #19 (mixerOpen) is fixed.

- "FM/OPL3" and "Phone" controls are added to mixer panel. "DirectX" is removed (because Windows NT emulates DirectX).

- Synchronization between mixer controls is fixed.

- Mixer default values were changed.

- Support HeadPhone for listening Audio.

- CD ROM volume function is changed.

- Limitation:

- Does not support WaveStream

- Mute does not function correctly on the BreakOut panel if play more than two files concurrently.

- Can not mute CD-ROM

- If version 0.01 is installed, it should be removed before installation of version 0.02. It will be fixed.

- Driver does not use MPU401 hardware, but logical MPU401 MIDI OUT device is present. All WHQL tests should be OK on this logical device. Please, use Multimedia Properties to select default MIDI device (FM/OPL3 or MPU401). Problem will be fixed next release.

- Legacy devices use default setting only:

| - Device | - I/O ports | - IRQ | - DMA |
| --- | --- | --- | --- |
| - FM/OPL3 | - 0x388-0x38B | - none | - none |
| - MPU401 | - 0x330-0x331 | - 9 | - none |
| - Joystick | - 0x201 | - none | - none |
| - SoundBlaster | - 0x220-0x22F | - 5 | - 1 |

- Default setting may cause conflicts with existing devices.

- It will be fixed in next releases.

- After installation of driver, Windows NT can not access to floppy drives.  This is the A0 hardware problem.

- Some WHQL tests fail, but it may be bugs in beta version of WHQL tests. We tried these tests on industry standard Yamaha OPL3 device (on Intel motherboard), and they fail too:

- DSound Drv Test #10 "Play sound file in primary [HW] buffer".

- DSound Drv Test #18 "Play in primary [HW] buffer, random sound file".

- MMSys Waveout Test #64 "waveOutGetVolume".

# Release 0.01

- Features/Bugs Fixed:

- Support PCM/Wave playback

- Provide PCM Volume controls via Mixer

- Limitation:

- Does not support FM/OPL3

- Does not support MPU401

- Does not support WaveStream

- Does not support PCM/Wave record

- Problem when installing on Gateway or Compaq