# Voice Answering Machine Setup for the Cirrus X2 Fax Voice CLM Modem

# Microsoft Phone

- On the Desktop, click on Start/Settings/Control Panel.

- Double Click on the Multimedia Icon

- In the Preferred Device List Box for both Playback and Record, select Voice Modem Wave #00 Handset.

- Set Preferred quality to Answering Machine Quality, and hit the Customize button.

- Set the format to Microsoft IMA ADPCM and click on OK.

- Check the Use preferred devices only checkbox at the bottom of the panel.

- At the top of this Multimedia Properties panel, click on the Advanced tab.

- Click on the + next to Audio Devices.

- Highlight (click on once) Audio for Cirrus CLM95 Voice Serial Wave Device.

- Click the Properties button at the bottom.

- Remove the check in the Do not map through this device checkbox and click on the Use audio features on this device selection.

- OK on all to exit.

- Close the Control Panel.

# Cheyenne Bitware with Voice

- Click on Setup/Modem Setup

- Verify that Modem is Cirrus Logic V.34+ (33.6) Voice Modem. If not, make that selection. Close this setup box. This will determine the correct parameters for the answering machine functionality.

- Click on Receive.

- Under Receive options, select Voice/Fax/Data to utilize all of the features of the modem.

- Close the setup.

# Pacific Image Communications, Inc. SuperVoice Pro 3.0 for Windows

- CLM Modem setup is determined by the installation.

Smith Micro Software, Inc. QuickLink MessageCenter III

1.     On the Desktop, click on Start/Settings/Control Panel.

- Double Click on the Multimedia Icon.

- In the Preferred Device List Box for both Playback and Record, select (None).

- UNcheck the Use preferred devices only checkbox at the bottom of the panel.

- Close the setup and Control Panel.