Cirrus Logic

Controller-Less Modem Release History

Version 1.024

History

Release 1.014

- First Official Release of the Comfire 1 code.  Supports Windows ’95 & ’98 and ITU V.90.

Release 1.015

- Never Released outside Cirrus Logic.

Release 1.016

- Never Released outside Cirrus Logic.

Release 1.017

- The Latest DSP V.90 (not Auto-Detection)!

- The second (and beyond) connection should now be as good as the first

- S91 & S92 control Data, & Fax Transmit Level.  Also S89, S90, & S93 control the DTMF High, DTMF Low, & Voice Transmit Level.  All Values are saved to EEProm when these commands are executed, and read from EEProm when powering up.

- V.42bis performance improved

- Fixed Voice Telephone Answering Machine bug (Hang-up after Recording a Message)

- Analog LoopBack Test Work (Currently a problem with V.34 at 2400).  Can not be supported at V.21, Bell103, & V.23 modes.

- Updated S-Registers to match "Comfire Programmer's Guide" (Note: S21, in the Programmer's Guide we have, show the value of 48 as default)

- We now connect in MNP mode.

- "at" <backspace> now shows the same ... before it was "aT"

- AT&Z is now fixed

- The Modem sends clear down before going On-Hook.

- Pulse dialing Make/Break reversal

- Fixed connect message when in ATX0.

- 1300-Hz Calling Tone Enabled with AT-C command

- AT-J controls V.42 Detection Phase

- Dial Modifier "Flash Hook" fixed

- ATZ works is on-line or in Test-Loop

- Cleaned up some "Uninstall" issues.

Release 1.018

- V.90 performance improved.

- MNP performance improved.

- Added a new test command AT*TT that allows manufactors to set the DSP to send a particular signal.  For command summary type “AT*TT?”, this is identical to the “AT$TT” command on the CL-MD565x and CL-MD345x Products.

- S23 now reports the Baudrate and Parity.

Release 1.019

- Never Released outside Cirrus Logic.

Release 1.020

- Never Released outside Cirrus Logic.

Release 1.021

- Bug fix:

- The modem would sometimes lock up if retraining

- Performance Issues:

- Selective retransmitting frames improvement.

- Utilizing more of the V.42bis implementation.

- Improvements:

- The retrain timer was extended for V.90 connects

- Added ATI20 - PCM information

- Added ATI21 - DSP Patch level (simple)

- Added ATI99 - DSP Patch level (extended)

Release 1.022

- Bug fix:

- Fax V.29 receive

- Fax transmit

- Relay chatter in switching from vce to fax

- Changes for ATI

- ALB in V.90 fixed to connect at highest rate

- Added missing V.90 rates

- +MS fix for max rate selection

- Removed improper use of some data commands while in voice mode

- Performance Issues:

- V.90 performance improved

- Improvements:

- TIES added

- AT01

- AT03 removed

- Support for X2 dropped

- ATH response time improved

- Able to connect V.32 with USR modems

Release 1.024

- Bug fix:

- ATI0, ATI9, ATI99 commands fixed

- WHQL bug fixes

- Performance Issues:

- V.90 performance improved

- Improvements:

- NT 4.0 and NT 5.0 drivers now supported